# Project Caesar: Live Emergency Demo Guide

This guide walks you through setting up a live, physical demonstration of Project Caesar on your Raspberry Pi 3. We will connect your phone camera to act as the AI eye, configure a mock gun for detection, and trigger both a physical ESP32 buzzer and a Twilio phone call to authorities.x

# Step 1: Using Your Phone Camera as the AI Eye

Since you are deploying everything on a Raspberry Pi 3, the easiest way to use your phone camera is to turn it into an IP webcam stream that the Pi can read.

Download the "DroidCam" or "IP Webcam" app on your smartphone from the App Store or Google Play.

Connect your phone and Raspberry Pi to the same WiFi network.

Open the app and tap "Start Server". It will give you a URL (e.g., http://192.168.1.50:8080/video).

Open the configs/edge-pi3.toml file on your Pi.

In the [optical] section, change the mode to "synthetic", or if you have DroidCam mapping to a Linux device, use sentinel mode. The easiest way without drivers is to use the python adapter. But assuming you just map it:
Change [sentinel] enabled = true and set device_id to your video URL string.

# Step 2: ESP32 Buzzer Setup (The Hardware Alarm)

We will use the MQTT Actuator. When Project Caesar detects the gun, it will broadcast an alert over your local WiFi. The ESP32 will listen for this alert and beep.

Connect a buzzer to your ESP32 (e.g., Positive to GPIO 13, Ground to GND).

Flash your ESP32 with a simple Arduino sketch (or ESPHome) that connects to your WiFi and connects to the Mosquitto MQTT broker running on your Pi (e.g., 192.168.x.x:1883).

Have the ESP32 subscribe to the topic: caesar/actions

In your Arduino code, when a message arrives, check if it contains the word "alert". If it does, drive GPIO 13 HIGH for 5 seconds to sound the buzzer.

Open configs/edge-pi3.toml and ensure the [actuators] section has the MQTT actuator enabled and pointing to your Pi's IP.

# Step 3: Configuring the Twilio Agentic Phone Call

When the threat is verified, the Python Orchestrator will make an automated text-to-speech call to your chosen phone number.

Open the file: /etc/systemd/system/caesar-orchestrator.service on your Hub machine (or Pi if running everything in hybrid).

Replace the placeholders in the Environment= lines with your real Twilio Account SID, Auth Token, Twilio Phone Number, and your real cell phone number (AUTHORITIES_PHONE).

Restart the orchestrator to load the keys: sudo systemctl daemon-reload && sudo systemctl restart caesar-orchestrator

Open services/mesh_orchestrator/alerts.py and UNCOMMENT lines 8-11 to activate the live webhook. Restart the orchestrator again.

# Step 4: Running the Live Demonstration

Start the edge node: ./target/release/uriel-edge-node --config configs/edge-pi3.toml

Point your phone camera at a clear scene. Stand in the frame.

Pull out your makeshift gun.

Watch the Magic Happen: The edge node will see the gun, the tracker will classify it as CRITICAL, the ESP32 buzzer will instantly start screaming, and your phone will ring with an automated voice call detailing your exact GPS coordinates.