Step 1: Install the Required Library

In the Arduino IDE, go to Sketch -> Include Library -> Manage Libraries, search for PubSubClient (by Nick O'Leary), and click Install. This is the library that allows the ESP32 to receive the MQTT messages.

Step 2: The Arduino Code

Copy this code, update the 3 network settings at the top, and flash it to your ESP32:

cpp

#include <WiFi.h>

#include <PubSubClient.h>

// 1. UPDATE THESE WITH YOUR NETWORK DETAILS

const char* ssid = "YOUR_WIFI_SSID";

const char* password = "YOUR_WIFI_PASSWORD";

const char* mqtt_server = "192.168.1.50"; // The IP address of your Raspberry Pi

// Pin configuration

const int BUZZER_PIN = 13;

WiFiClient espClient;

PubSubClient client(espClient);

// This function runs whenever a new message arrives on "caesar/actions"

void callback(char* topic, byte* payload, unsigned int length) {

// Convert the incoming payload to a readable string

String message = "";

for (int i = 0; i < length; i++) {

message += (char)payload[i];

}

Serial.print("Message arrived on topic: ");

Serial.println(topic);

Serial.print("Payload: ");

Serial.println(message);

// Check if the JSON payload contains the word "alert"

if (message.indexOf("\"action\":\"alert\"") != -1 || message.indexOf("\"action\": \"alert\"") != -1) {

Serial.println("🚨 CAESAR ALERT TRIGGERED! SOUNDING ALARM! 🚨");

// Turn the buzzer ON

digitalWrite(BUZZER_PIN, HIGH);

// Keep it on for 5 seconds

delay(5000);

// Turn the buzzer OFF

digitalWrite(BUZZER_PIN, LOW);

}

}

void setup_wifi() {

delay(10);

Serial.println();

Serial.print("Connecting to ");

Serial.println(ssid);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nWiFi connected.");

Serial.print("IP address: ");

Serial.println(WiFi.localIP());

}

void reconnect() {

// Loop until we're reconnected to the MQTT broker

while (!client.connected()) {

Serial.print("Attempting MQTT connection...");

// Create a random client ID

String clientId = "ESP32Client-";

clientId += String(random(0xffff), HEX);

// Attempt to connect

if (client.connect(clientId.c_str())) {

Serial.println("connected");

// Subscribe to the Caesar Actuator topic

client.subscribe("caesar/actions");

} else {

Serial.print("failed, rc=");

Serial.print(client.state());

Serial.println(" try again in 5 seconds");

delay(5000);

}

}

}

void setup() {

Serial.begin(115200);

pinMode(BUZZER_PIN, OUTPUT);

digitalWrite(BUZZER_PIN, LOW); // Ensure buzzer is off on boot

setup_wifi();

// Configure the MQTT broker details

client.setServer(mqtt_server, 1883); // 1883 is the default Mosquitto port

client.setCallback(callback);

}

void loop() {

if (!client.connected()) {

reconnect();

}

client.loop(); // Keeps the MQTT connection alive

}